N+ Network Security

Day 5

1. Simulate a DNS resolution failure on your local machine by editing the hosts file to point [www.spotify.com](http://www.spotify.com) to a wrong IP address, then try to ping the domain and document the error message you receive.

= Simulate DNS Resolution Failure (Short)

- Open the hosts file:

- C:\Windows\System32\drivers\etc\hosts

- Add this entry:

- 192.168.1.99    www.spotify.com

- Save the file and run:

- ping www.spotify.com

2. Configure a DHCP server in a network simulator (such as Cisco Packet Tracer or GNS3) and intentionally set an incorrect default gateway; connect a client device and verify the assigned gateway, then describe how you would identify and fix this issue.

= DHCP Incorrect Gateway Configuration (Short)

- Configure the DHCP server with an incorrect default gateway (e.g., 192.168.1.254 instead of 192.168.1.1).

- Connect a PC and obtain an IP address using DHCP.

- Verify the assigned gateway:

- ipconfig

- The PC will show the incorrect gateway (192.168.1.254) and will not access other networks.

3. Given a scenario where devices on your simulated network are unable to access websites by name but can by IP address, use the nslookup or dig tool to diagnose the DNS problem and write down the troubleshooting steps and solution.

= DNS Troubleshooting (Short)

- Test DNS:

- nslookup www.google.com

(or dig www.google.com on Linux)

- If DNS fails but IP works, check the DNS server address:

- ipconfig /all

- Verify the DNS server is running and the DNS records are correct.

- Update the client to use the correct DNS server and renew the IP:

- ipconfig /renew

- ipconfig /flushdns

4. A device in your simulated network is not receiving an IP address from the DHCP server and shows an APIPA address (169.x.x.x). List the possible causes for this and document the step-by-step process you would use to resolve it.<br><br><em><strong>Hint:</strong> Check for network connectivity, DHCP scope exhaustion, and server availability.</em>

= APIPA (169.x.x.x) Troubleshooting (Short)

Possible Causes:

- DHCP server is down.

- DHCP scope has no available IP addresses.

- Network cable or connection issue.

- Incorrect DHCP configuration.

Steps to Resolve:

- Check the network cable/connection.

- Verify the DHCP server is running.

- Check that the DHCP scope has available IP addresses.

- Renew the IP address:

- ipconfig /release

- ipconfig /renew

- Verify the new IP using:

- ipconfig

5. Create a table documenting at least three common DNS issues and three common DHCP issues you encountered or researched, including symptoms, possible causes, and recommended solutions for each.

=

| Issue Type | Symptom | Possible Cause | Recommended Solution |
| --- | --- | --- | --- |
| DNS | Website name not resolving | Wrong DNS server address | Configure the correct DNS server |
| DNS | nslookup fails | DNS service is stopped | Restart the DNS server/service |
| DNS | Wrong IP returned | Incorrect DNS record | Update the DNS zone records |
| DHCP | APIPA address (169.x.x.x) | DHCP server unavailable | Start the DHCP server and renew the IP |
| DHCP | No IP assigned | DHCP scope exhausted | Increase the DHCP address pool |
| DHCP | No internet after getting an IP | Incorrect default gateway | Correct the gateway in DHCP settings and renew the IP |